BÀI TẬP 2:

1. What is the decimal value of the following single-precision floating-point numbers?

a) 1010 1101 0001 0100 0000 0000 0000 0000 (binary)

b) 0100 0110 1100 1000 0000 0000 0000 0000 (binary)

2. Show the IEEE 754 binary representation for: -75.4 in …

a) Single Precision

b) Double precision

3.       x = 1100 0110 1101 1000 0000 0000 0000 0000 (binary) and

y = 0011 1110 1110 0000 0000 0000 0000 0000 (binary)

are single-precision floating-point numbers.

Perform the following operations showing all work:

a) x + y

b) x * y

4.         x = 0101 1111 1011 1110 0100 0000 0000 0000 (in binary) and

y = 0011 1111 1111 1000 0000 0000 0000 0000 (in binary) and

z = 1101 1111 1011 1110 0100 0000 0000 0000 (in binary)

represent single precision IEEE 754 floating-point numbers.

Perform the following operations. showing all work:

a) x + y

b) Result of (a)+ z

c) Why is the result of (b) counterintuitive?

5.   IA-32 offers an 80-bit extended precision option with a 1 bit sign, 16-bit exponent, and 63-bit fraction (64-bit significand including the implied 1 before the binary point). Assume that extended precision is similar to single and double precision.

a) What is the bias in the exponent?

b) What is the range (in absolute value) of normalized numbers that can be represented by the extended precision option?

6.   We wish to compare the performance of two different computers: M1 and M2. The

following measurements have been made on these computers:

a) Which computer is faster for each program, and how many times as fast is it?

b) Find the instruction execution rate (instructions per second) for each computer

when running program 1.

c) The clock rates for M1 and M2 are 3 GHz and 5 GHz respectively. Find the CPI

for program 1 on both machines.

d) Suppose that program 1 must be executed 1600 times each hour. Any remaining

time should used to run program 2. Which computer is faster for this workload?

Performance is measured here by the throughput of program 2.

7. Suppose you wish to run a program P with 7.5 × 109  instructions on a 5 GHz machine with a CPI of 1.2.

a) What is the CPU execution time?

b) When you run program P, it takes 3 seconds of wall time to complete. What is

the percentage of the CPU time program P received?

8. Consider two different implementations, M1 and M2, of the same instruction set.

There are five classes of instructions (A, B, C, D and E) in the instruction set. M1 has a

clock rate of 4 GHz and M2 has a clock rate of 6 GHz.

a) Assume that peak performance is defined as the fastest rate that a computer

can execute any instruction sequence. What are the peak performances of M1 and M2

expressed in instructions per second?

b) If the number of instructions executed in a certain program is divided equally

among the classes of instructions, except that for class A, which occurs twice as often

as each of the others, how much faster is M2 than M1?

9. Consider two different implementations, M1 and M2, of the same instruction set.

There are three classes of instructions (A, B, and C) in the instruction set. M1 has a clock

rate of 6 GHz and M2 has a clock rate of 3 GHz. The CPI for each instruction class on

M1 and M2 is given in the following table:

The above table also contains a summary of the usage of instruction classes generated by

three different compilers: C1, C2, and C3. Assume that each compiler generates the same

number of instructions for a given program.

a) Using C1 compiler on both M1 and M2, how much faster is M1 than M2?

b) Using C2 compiler on both M1 and M2, how much faster is M2 than M1?

c) If you purchase M1, which compiler would you use?

d) If you purchase M2, which compiler would you use?

e) Which computer and compiler combination give the best performance?

10.  Consider the following fragment of MIPS code. Assume that a and b are arrays of words and the base address of a is in $a0 and the base address of b is in $a1. How many

instructions are executed during the running of this code? If ALU instructions (addu and addiu) take 1 cycle to execute, load/store (lw and sw) take 5 cycles to execute, and the branch (bne) instruction takes 3 cycles to execute, how many cycles are needed to execute the following code (all iterations). What is the average CPI?

addu 	$t0, $zero, $zero 		# i = 0

addu 	$t1, $a0, $zero 		# $t1 = address of a[i]

addu 	$t2, $a1, $zero 		# $t2 = address of b[i]

addiu 	$t3, $zero, 101 		# $t3 = 101 (max i)

loop:   	        lw 		$t4, 0($t2) 		 	# $t4 = b[i]

addu 	$t5, $t4, $s0 		 	# $t5 = b[i] + c

sw 	$t5, 0($t1) 		 	# a[i] = b[i] + c

addiu 	$t0, $t0, 1 		 	# i++

addiu 	$t1, $t1, 4 		 	# address of next a[i]

addiu 	$t2, $t2, 4 		 	# address of next b[i]

bne 	$t0, $t3, loop 		 	# loop if (i != 101)